REPORT for Experiment 9

Experiment Title：

Methods of counting micobes-total cell counts

Instructor(s)：

Pinmei Wang

Lab Location：

海工楼

Date Performed：

2024.11.16

1. Introduction

(1)Quantitative confirmation of microalgae by Hemocytometer

①Hemocytometer

The hemocytometer contains nine cells, twenty-five cells in each of the central crosses, and sixteen cells in each of the four corners.

②Counting cells with a hemocytometer under microscope

Depending on the sample density, select a medium or small cell for counting. When counting, take the left and upper sides of the sample and discard the right and lower sides for the sample on the margins. When counting, pay attention to the serpentine count, that is, each count cell and the next one are consecutive. The data is recorded after the count is complete.

③Calculated the cell member of microscope

The number of cells per cell can be calculated by multiplying the average number of cells per cell by the dilution rate and dividing by the volume per cell

- Use spectrophotometer to calculate

OD600 is a common method for estimating the concentration of bacterial or other cells in a liquid.OD600 is perferable to UV spectroscopy when measuring the growth over time of a cell population because at this wavelength, the cells will not be killed as they would under too much UV light.

2. Experimental Purpose

(1)Understand how a Neubauer Hemocytometer is used to count cells

(2)Be able to perform a total cell count using the chamber

(3)Know how to use spectrophotometer to measure OD of microbial cells

(4)Be able to calcute the cell concentration from the record of detection the cell number by Neubauer  Hemocytometer and spectrophotometer methods

3. Equipment and Materials

(1)Fungal cell suspension in a tube

(2)Neubauer chamber and coverslip

(3)Microscope

(4)Sterile saline and bottles

(5)Pipette and tips

(6)spectrophotometer

4. Experimental Procedure

5. Record and Results

Neubauer Haemocytometer

- Did you count in the 4 × 4 large squares, or in the 5 × 5 small central squares? And in the squares you choose, how many cells you count in total?

We count in the 5 × 5 small central squares,there are 12 cells in 5 small central squares

- What is the volume of 4 large squares in total (“1”-“4” as shown in Fig 9.1B) in mL? What is the volume of the central square (5 × 5) in mL?

4 large squares in total:4 x 1 mm x 1 mm x 0.10 mm=0.4mm3=0.4mL

central square (5 × 5)=1 mm x 1 mm x 0.10 mm=0.1mm3=0.1mL

- According to the number of the cells you count and the volume of the squares you choose, show your calculations to determine the number of cells per ml in the suspension.

We count in the 5 × 5 small central squares,there are 12 cells in 5 small central squares.So

There should be 12 x 5 x 10 x 1000=600000cells per mL

Spectrophotometer

- What is the number of cells/mL as estimated using the spectrophotometer?

The final data we get is 0.801.

Y=107 x OD600-907614=7102386 cells/mL

6. Discussion

1) Which is the direct method for determining cell numbers?

Hemocytometer is the direct method for determining cell numbers.

- What is the basic difference between the cell counts obtained from the viable count method and the total count methods?

Viable Count: Measures only live cells, useful for assessing microbial activity.

Total Count: Measures all cells (live and dead), useful for a comprehensive population count.